MCQ

1.a
2.c
3.d
4.a

(Reply with your choices like: 1-a, 2-b, 3-c … )

✅ True /

5. True or False: When installing a new hard drive, it is essential to format it before use.
➡️ True — A new HDD/SSD must be partitioned & formatted to store data.

6. True or False: A POST error indicates a problem with the CPU.
➡️ False — POST errors can be due to RAM, GPU, motherboard, keyboard, etc.

7. True or False: It is safe to remove a USB flash drive without ejecting it first.
➡️ False — It can cause data corruption. Always “Eject safely”.

✍️ 8. Steps to Install a New Graphics Card in a Desktop Computer

- Power off the computer and unplug the PSU.

- Open the cabinet by removing screws.

- Locate the PCIe x16 slot on the motherboard.

- If an old GPU is present, unscrew and remove it.

- Insert the new graphics card firmly into the PCIe slot.

- Secure the card to the case with screws.

- Connect PCIe power cables from the PSU (6-pin/8-pin).

- Close the case and plug in the computer.

- Connect monitor to the GPU output (HDMI/DP).

- Boot the PC and install GPU drivers (NVIDIA/AMD).

✍️ 9. What is RAID? Explain common RAID levels

RAID (Redundant Array of Independent Disks)
A storage technology that uses multiple drives to increase:

- Performance

- Fault tolerance

- Storage capacity

Common RAID configurations:

RAID 0 — Striping
✔ Fast performance
✘ No redundancy (if one disk fails, all data lost)

RAID 1 — Mirroring
✔ Full data redundancy
✘ Storage cost doubled (same data stored on both disks)

RAID 5 — Striping with Parity
✔ Good performance
✔ Single-disk fault tolerance
✘ Requires at least 3 disks

RAID 10 — Combination of RAID 1 + RAID 0
✔ High performance + redundancy
✘ Requires at least 4 disks

✍️ 10. How to Replace a CPU Fan in a Desktop Computer

- Turn off the PC and disconnect power.

- Open the cabinet.

- Disconnect the CPU fan cable from the motherboard.

- Remove the heat sink and fan assembly by releasing clips/screws.

- Clean the CPU surface using isopropyl alcohol.

- Apply a pea-sized drop of thermal paste.

- Install the new CPU fan + heat sink on top of the CPU.

- Secure the fan using clips or screws.

- Plug the CPU_FAN header into the motherboard.

- Close the case and power on the PC to test fan rotation.

✍️ 11. Importance of Regular Maintenance for Computer Hardware

Regular maintenance:

- Improves performance

- Extends hardware lifespan

- Prevents overheating

- Reduces system crashes

- Ensures clean and stable electrical connections

Examples of maintenance tasks:

- Cleaning dust from fans, heat sinks, and vents

- Checking and tightening loose cables

- Reapplying thermal paste after a few years

- Updating BIOS and drivers

- Running disk cleanup and defragmentation

- Checking PSU voltages

- Cleaning keyboard/mouse

- Monitoring CPU & GPU temperatures